| Layer | Description | Unit |
| --- | --- | --- |
| DelaySum | - Sum of 15 minute time bin delay for an average weekday - Delay = Estimated volume * Additional Travel Time - 15 Minute volume is estimated by AADT and a historical hourly volume factor by functional class - Additional travel time = Travel time at median speed – Travel time at 85% of free flow speed - Free flow speed is estimated by median speed from weekdays: 10 pm - 5 am | vehicle-hour |
| WDDelayAM | - Sum of AM peak period delay - AM peak period is weekdays 6 to 10 am (4 hours) | vehicle-hour |
| WDDelayMid | - Sum of mid-day time period delay - Mid-day is weekdays 10 am to 4 pm (6 hours) | vehicle-hour |
| WDDelayPM | - Sum of PM peak period delay - PM peak period is weekdays 4 to8 pm (4 hours) | vehicle-hour |
| PTI_AMP | - Planning Time Index (PTI) for AM peak period - PTI = 95th Percentile Travel TimeFFS Travel Time - Planning time means the total travel time, which includes extra time (buffer time) required by travelers to ensure a high rate of on-time arrival. | - |
| PTI_MIDD | - PTI for mid-day period | - |
| PTI_PMP | - PTI for PM peak period | - |
| PTI_Peak | - PTI for AM and PM peak period (Weekdays: 6-10 am and 4-8 pm) | - |
| PTI_WE | - PTI for Weekend | - |
| PTI_Flag | - Segment should not have a PTI result less than 1, but, some calculated result was less than 1 because free flow speed was underestimated due to data limitation such as lack of number of observation. In this case, PTI results is adjusted to 1. - PTI_Flag = 1 means at least one of PTI result for this segment had less than 1 and this segment PTI number is adjusted. | - |
| pkAM6_10 | - Average congestion percentage in weekdays 4-hour AM peak period (6 to 10 AM) | Percentage |
| pkPM4_8 | - Average congestion percentage in weekdays 4-hour PM peak period (4 to 8 PM) | Percentage |
| pkAM7_9 | - Average congestion percentage in weekdays 2-hour AM peak period (7 to 9 AM) | Percentage |
| pkPM5_7 | - Average congestion percentage in weekdays 2-hour PM peak period (5 to7 PM) | Percentage |
| md10AM_4PM | - Average congestion percentage in weekdays 6-hour mid-day period (10 AM to 4 PM) | Percentage |
| eve8PM_6AM | - Average congestion percentage in weekdays 10-hour evening/night/morning period (8 PM – 6 AM) | Percentage |
| md9AM_5PM | - Average congestion percentage in weekdays 8-hour mid-day period (9 AM to 5 PM) | Percentage |
| eve7PM_7AM | - Average congestion percentage in weekdays 12-hour evening/night/morning period (7 PM to 7 AM) | Percentage |

* If SPM_ID segment does not have any available data, value is displayed as “-1”.